## 1. System Overview

### 1.1 System Functions (GUI Scope)

The GUI shall provide the following system-level capabilities:

- Real-time 2D/3D visualization of swarm maps and robot states

- Interactive task creation, editing, and reassignment

- Real-time monitoring of robot health and sensor states

- Visual control of swarm behaviors and formations

- Camera feed display and switching

- Emergency override and teleoperation interfaces

- User feedback, alerting, and system state indication

- Simulation environment loading, execution, and control via Webots

- Robot selection, command confirmation, and command dispatch to simulation and real robots

## 2. Simulation & GUI Execution Environment

### 2.1 Simulation Launch and Integration

- The GUI shall provide a simulation selection interface prior to Webots execution.

- Upon simulation selection, the GUI shall display a configuration popup allowing the user to:

- Select the simulation world

- Select the number of robots

- Select enabled sensors

- The GUI shall launch Webots using the selected configuration parameters.

- The GUI shall run in a dedicated application window while Webots executes in a parallel window or tab.

- The GUI shall automatically connect to the ROS2 simulation namespace after Webots startup.

### 2.2 Environment Loading

- The GUI shall allow users to:

  - Select simulation environments from a file browser

  - Reload environments during runtime

  - Reset simulation state

- The GUI shall display simulation connection status and environment metadata.

### 2.3 Robot Selection and Command Confirmation

- The GUI shall require explicit robot selection before issuing any command.

- The GUI shall support:

  - Single robot selection

  - Multi-robot selection

The GUI shall require user confirmation before transmitting:

  - Task updates

  - Behavior changes

  - Sensor toggles

  - Emergency commands

- The GUI shall visually display the list of affected robots before confirmation.

## 3. GUI Functional Requirements

## 3.1 Tactical Mapping & Swarm Visualization Module

### 3.1.1 Swarm Merged Map Display

- The GUI shall visualize a unified 2D/3D occupancy grid map derived from swarm-level SLAM outputs.

- The GUI shall support switching between:

  - Individual robot maps

  - Global merged maps

  - Costmap overlays

- The GUI shall allow toggling of map layers including obstacles, inflation zones, and explored areas.

### 3.1.2 Target, Feature, and Landmark Visualization

- The GUI shall depict detected targets and landmarks using standardized icons.

- Each detected entity shall display:

  - Unique identifier

### 3.1.3 Navigation and View Controls

- The GUI shall provide interactive map zooming and rotation controls.

- The GUI shall include controls for:

  - Fitting the entire map into view

  - Resetting map orientation to default alignment

### 3.1.4 Robot Avatar Representation

- Each robot shall be represented by a unique icon showing:

  - Current position

  - Velocity vector

  - Robot identifier label

### 3.1.5 Multi-Robot Selection and Grouping

- The GUI shall allow single-robot selection via click interaction.

- The GUI shall support multi-robot selection using bounding box or modifier keys.

- The GUI shall allow creation of logical robot groups for task assignment and behavior control.

## 3.2 Multi-Robot Task Management Interface

### 3.2.1 Task Assignment

- The GUI shall allow operators to assign one or more robots to a task or work zone.

- The GUI shall display task attributes including:

  - Task ID

  - Assigned robots

  - Status (Pending, Active, Completed, Failed)

- The GUI shall allow modification of assigned robots during task execution.

### 3.2.3 On-the-Fly Task Amendment

- The GUI shall allow tasks to be paused, resumed, and cancelled during mission execution.

- The GUI shall support live reassignment of robots between tasks without restarting the mission.

- Task modifications shall be visually reflected immediately in the task management panel.

## 3.3 Unified Sensor and Health Monitoring Interface

### 3.3.1 Individual Robot Health Display

- The GUI shall display real-time health indicators for each robot including:

  - Battery percentage

  - Network connectivity

- The GUI shall present health indicators using standardized color-coded thresholds.

### 3.3.2 Fleet-Level Health Summary

- The GUI shall provide a fleet summary panel displaying:

  - Total robots online/offline

  - Average battery level

  - Number of robots in warning/critical states

  - Overall communication health

- The GUI shall support filtering robots by health status.

### 3.3.3 Sensor Status Display

- The GUI shall display operational status for each onboard sensor including:

  - LiDAR

  - GPS

  - IMU

  - Cameras

- Each sensor indicator shall show:

  - Enabled/disabled state

  - Last update timestamp

  - Health or fault state

### 3.3.4 Sensor Power Control Interface

- The GUI shall provide a dedicated interface for enabling and disabling sensors on selected robots.

- The GUI shall display confirmation dialogs for sensor power state changes.

- The GUI shall visually indicate sensor state transitions and command acknowledgment.

## 3.4 Camera and Vision Interface

### 3.4.1 Multi-Camera Feed Display

- The GUI shall support real-time display of camera streams from multiple robots.

- The GUI shall allow selection between five camera feeds per robot:

  - Front

  - Rear

  - Left

  - Right

### 3.4.3 Camera Selection and Routing

- The GUI shall allow selecting camera feeds per robot independently.

- The GUI shall allow switching camera feeds without interrupting other GUI operations.

## 3.5 Swarm Behavior and Formation Control Interface

### 3.5.1 Flocking Behavior Control

- The GUI shall provide controls to configure flocking parameters.

- The GUI shall display current parameter values and allow reset to default.

### 3.5.3 Pattern Formation(Forging) Control

- The GUI shall allow operators to command swarm-level formations.

## 3.6 Operational Modes and Teleoperation Interface

### 3.6.1 Mode Indication

- The GUI shall clearly display the current operational mode:

  - Autonomous

  - Manual (Emergency)

- Mode indicators shall be persistent and visible at all times.

### 3.6.3 Emergency Stop Interface

- The GUI shall provide a globally visible Emergency Stop (E-Stop) control.

- The E-Stop shall:

  - Be visually distinct

  - Require confirmation

  - Trigger immediate UI alert state

- The GUI shall provide visual feedback confirming E-Stop activation and release.

### 3.6.4 Robot-Level Emergency Control

- The GUI shall provide robot-specific stop controls.

### 3.6.5 Teleoperation Control Interface

- The GUI shall provide joystick-style or keyboard-based teleoperation widgets.

- Teleoperation controls shall display:

  - Commanded linear velocity

  - Commanded angular velocity

- Teleoperation shall be limited to emergency or parking operations.

## 5. GUI Interface and Message Display Specification

### 5.1 ROS2 Message Consumption (GUI Side)

| Module | ROS Topic | Data Type | GUI Usage |
| --- | --- | --- | --- |
| Mapping | /swarm_map | nav_msgs/OccupancyGrid | Map rendering |
| Localization | /robot_pose | geometry_msgs/PoseStamped | Robot avatar position |
| Tasking | /swarm/task_amend | TaskUpdate.msg | Task panel update |
| Health | /robot/health | HealthStatus.msg | Health dashboard |
| Vision | /robot/camera_[1–5] | sensor_msgs/Image | Camera viewer |
| Control | /teleop/cmd_vel | geometry_msgs/Twist | Teleop display |

### 5.2 GUI Command Publishing (Operator Actions)

| GUI Action | ROS Topic | Message Type |
| --- | --- | --- |
| Task creation | /swarm/task_create | TaskCommand.msg |
| Task update | /swarm/task_update | TaskCommand.msg |
| Sensor toggle | /robot/sensor_control | SensorCommand.msg |
| Mode switch | /system/mode_set | ModeCommand.msg |
| Emergency stop | /system/e_stop | std_msgs/Bool |

## 6. Simulation Interface Requirements (Webots Integration)

### 6.1 Simulation Startup

• The GUI shall provide a simulation selection interface through the Simulation Control panel.
• Upon simulation selection, the GUI shall display a configuration popup allowing the user to select the world, number of robots, and active sensors.
• The GUI shall launch Webots using the selected configuration parameters.
• The GUI shall verify simulation connectivity before enabling mission controls.

### 6.2 Simulation Environment Control

- The GUI shall allow users to:

  - Load simulation environments

  - Reset simulation

- Environment loading shall require confirmation before execution.

### 6.4 Robot Selection and Command Confirmation (Simulation & Real Robots)

- The GUI shall require robot selection before any command is issued.

- The GUI shall present a confirmation dialog showing:

  - Selected robots

  - Command type

  - Expected effect

- The GUI shall transmit commands only after user confirmation.

### 6.5 Simulation-to-Robot Transition

- The GUI shall allow seamless switching between simulation mode and real-robot mode.

- The GUI shall clearly indicate whether commands are being sent to simulation or physical robots.